CHAPTER 17

NONRENEWABLE ENERGY RESOURCES

THINKING

Goals

See bulleted list of questions on p. 251 of text.

Objectives

1.	How much of the total energy used to heat the earth and the earth’s buildings comes from commercial energy? List five key questions to ask about each energy alternative to evaluate energy resources. Define net energy, and state its significance in evaluating energy resources.

2.	Distinguish among primary, secondary, and tertiary oil recovery. List the advantages and disadvantages of using conventional oil, oil from oil shale, and oil from tar sands to heat space and water, produce electricity, and propel vehicles.

3.	Distinguish among natural gas, liquefied petroleum gas, liquefied natural gas, and synthetic natural gas. List the advantages and disadvantages of using natural gas as an energy source.

4.	List and describe three types of coal. Indicate which is preferred for burning and which is most available. List and briefly describe three methods for extracting coal. List advantages and disadvantages of using coal as a fuel source.

5.	Briefly describe the components of a conventional nuclear reactor. List advantages and disadvantages of using conventional nuclear fission to create electricity. Be sure to consider the whole nuclear fuel cycle, including disposal of radioactive wastes, safety and decommissioning of nuclear power plants, and the potential for proliferation of nuclear weapons.

6.	Summarize current thinking about disposal of low-level and high-level radioactive wastes.

7.	List and briefly describe three ways to decommission a nuclear power plant. List findings of a 1987 commission that brought the credibility of the Nuclear Regulatory Commission to safeguard the nuclear power industry into question.

8.	Describe the potential use of breeder nuclear fission and nuclear fusion as energy sources.

Key Terms  (Terms are listed in the same font as they appear in the text.)

Chernobyl (p. 350)

solar capital (p. 351)

commercial energy (p. 351)

biomass energy (p. 352)

fuelwood shortage (p. 352)

high-quality energy (p. 354)

net energy (p. 354)

automatically wasted (p. 354)

unnecessarily wasted (p. 354)

useful energy (p. 354)

net energy ratio (p. 354)

nuclear fuel cycle (p. 355)

petroleum (p. 355)

crude oil (p. 355)

conventional (light) oil (p. 355)

heavy crude oil (p. 355)

refinery (p. 355)

petrochemicals (p. 356)

oilaholics (p. 356)

oil sand (p. 360)

tar sand (p. 360)

bitumen (p. 360)

oil shale (p. 361)

kerogen (p. 361)

shale oil (p. 361)

natural gas (p. 362)

conventional natural gas (p. 362)

unconventional natural gas (p. 362)

methane hydrate (p. 362)

liquefied petroleum gas (LPG) (p. 362)

liquefied natural gas (LNG) (p. 362)

coal (p. 364)

area strip mining (p. 364)

contour strip mining (p. 364)

synthetic natural gas (SNG) (p. 365)

coal gasification (p. 365)

coal liquefaction (p. 365)

synfuels (p. 365)

light-water reactors (LWRs) (p. 366)

core (p. 366)

uranium oxide fuel (p. 366)

control rods (p. 366)

moderator (p. 366)

pressurized water reactors (p. 366)

coolant (p. 366)

containment vessel (p. 366)

water-filled pools (p. 366)

dry casks (p. 366)

high-level radioactive wastes (p. 367)

decommissioned (p. 367)

closed nuclear fuel cycle (p. 367)

open nuclear fuel cycle (p. 368)

Three Mile Island (p. 369)

low-level radioactive wastes (p. 372)

Challenger (p. 372)

Yucca Mountain (p. 373)

dirty radioactive bombs (p. 375)

advanced light-water reactors (ALWRs) (p. 376)

passive safety features (p. 376)

breeder nuclear fission reactor (p. 376)

Outline

17-1 	Evaluating Energy Resources

- Ninety-nine percent of the energy that heats the earth and all of our buildings comes directly from the sun.

1.	Without the sun’s energy, life on earth wouldn’t exist. The sun is a giant nuclear fusion reactor.

2.	The sun provides other indirect forms of renewable solar energy such as wind, falling/flowing water, and biomass.

3.	Commercial energy sold in the marketplace makes up the remaining 1% of the energy we use, most from nonrenewable resources.

B.	About 84% of commercial energy consumed in the world comes from nonrenewable energy sources (fossil fuels, nuclear energy, etc.).

1.	About 50% of people in developing countries burn wood and charcoal to heat dwellings and cook.

2.	Biomass energy is renewable as long as it is not harvested faster than it can be replenished.

3.	Many people in developing countries face a fuelwood shortage that is getting worse because of unsustainable harvesting of fuelwood.

4.	Premature deaths are also caused from breathing particles emitted by burning wood indoors in open fires.

C.	People in the U.S. consume as much energy in a day as a person in the poorest countries consumes in a year.

1.	In 2004, the U.S. used 24% of the world’s commercial energy with only 4.6% of the population.

2.	India has 16% of the world’s population and consumes about 3% of the world’s commercial energy.

3.	About 94% of the commercial energy in the U.S. comes from nonrenewable energy resources, with the remaining 6% coming from renewable biomass and hydropower.

4.	An important environmental, economic, and political issue is what energy resources the U.S. might be using by 2050 and 2100.

5.	Burning fossil fuel causes more than 80% of U.S. air pollution and 80% of CO2 emissions. Many energy experts feel the need to move to cleaner energy resources, not because of energy source depletion but to clean up the environment.

6.	Fossil fuel and nuclear power industries have been receiving government subsidies and do not want to give them up. The energy path for the U.S. is primarily a political decision.

D.	We need to answer several questions in deciding which energy resources to promote.

1.	It takes at least 50 years and huge investments to phase in new energy alternatives to where they provide 10–20% of total energy use.

2.	The following questions must be answered for each alternative:

a.	How much of the energy resource is likely to be available in the near future and in the long term?

b.	What is the net energy yield for the resource?

c.	How much will it cost to develop, phase in, and use the resource?

d.	What government research and development subsidies and tax breaks will be used to help develop the resource?

e.	How will dependence on the resource affect national and global economic and military security?

f.	How vulnerable is the resource to terrorism?

g.	What are the affects of extracting, transporting, and using the resource on human health and the earth’s climate?

E.	Net energy is the amount of high-quality usable energy available from a resource after subtracting the energy needed to make it available for use.

1.	It takes energy to get energy.

2.	Net energy available for use is calculated by estimating total energy available minus energy used, automatically wasted, and unnecessarily wasted in finding, processing, concentrating, and transporting to users. Figure 17-7 shows estimated net energy ratios for various types of processes.

17-2 	Oil

A.	Crude oil is a thick, gooey liquid consisting of hundreds of combustible hydrocarbons mixed with small amounts of sulfur, oxygen, and nitrogen impurities.

1.	Three geological events led to the presence of oil.

a.	Sediments buried organic material faster than it could decay.

b.	Sea floors with these sediments were subjected to the right pressure and heat to convert organic material to oil.

c.	Oil collected in porous limestone or sandstone and was capped by shale or silt to keep it from escaping.

2.	Oil and natural gas provide us with food grown with the help of hydrocarbon-based fertilizers and pesticides. This type of oil is known as conventional oil or light oil.

3.	The oil industry today is a marvel of high tech events to extract, refine, market, and distribute to the world’s populations.

4.	Oil and natural gas are often found together under a dome. On average, only about 40–50% of the oil in the deposit is recovered.

5.	The remaining heavy crude oil is too difficult or expensive to extract.

6.	Drilling causes only moderate environmental damage, but transportation around the world results in oil spills on land and in aquatic systems. Harmful effects are also associated with extraction, processing, and use of any nonrenewable resource from the earth’s crust.

7.	Improved extraction technologies should cause less environmental damage.

8.	Crude oil is transported to a refinery where it is broken down into components with different boiling points. This process accounts for about 8% of all U.S. energy consumption.

9.	Petrochemicals are oil distillation products that are used as raw materials in manufacturing pesticides, plastics, synthetic fibers, paints, medicines, and other products.

B.	Eleven OPEC countries have 78% of the world’s proven oil reserves.

1.	The control of current and future oil reserves is the single greatest source of global economic and political power.

2.	Saudi Arabia has the largest supply of oil reserves with 25%. The OPEC nations are almost all in the Middle East. It is thought that their production of global oil will increase from 30% at present to 50% in the future.

3.	Oil is the most widely used resource in the world. It is of concern that the oil producing areas are politically stable. This area is often very volatile and may be subject to terrorist attacks.

C.	The U.S. has only 2.9% of the world’s proven oil reserves and about one-fourth of that comes from offshore drilling and from Alaska’s North Slope.

1.	The U.S. uses about 26% of crude oil extracted worldwide each year.

2.	U.S. oil extraction has declined since 1985. Most of the oil extracted costs $7.50–8.00/barrel compared to about $2.50/barrel from Saudi Arabia.

3.	In 2003, the U.S. imported about 55% of the oil it used. By 2020, the U.S. could be importing 64–70% of the oil it will use.

4.	Some analysts feel that importing oil is not all bad, that U.S. oil reserves should be held in reserve.

D.	Known and projected global reserves should last 42–93 years, and U.S. reserves for 10–48 years depending on how rapidly we use oil.

1.	Oil production is expected to peak sometime between 2010 and 2030.

2.	Oil will become increasingly more expensive.

E.	Use of conventional oil at current rates means we need to discover oil reserves equal to a new Saudi Arabian supply every 10 years.

1.	We ignore the exponential growth in use of oil.

2.	If oil continues to be used at current rates:

a.	Saudi Arabia could supply world oil needs for about 10 years.

b.	Estimated Alaskan North Slope oil reserves would meet current demands for 6 months or U.S. demands for 3 years.

c.	Alaska’s Artic National Wildlife Refuge would meet demands for 1–5 months or U.S. oil demand for 7–24 months.

3.	Many developing countries, such as China and India, are rapidly expanding their use of oil.

4.	If everyone in the world used as much oil as the average American, the world’s proven reserves would be gone in a decade.

F.	The Arctic National Wildlife Refuge (ANWR) is a very fragile ecosystem. A diverse community of species call this tundra biome home. It is the only stretch of Alaska’s arctic coastline not open to oil and gas development.

1.	U.S. oil companies have been trying to explore this area for oil and gas. They are supported by Alaska’s elected officials, so they can use the revenue obtained to finance the state budget and give dividends to its citizens.

2.	Environmentalists feel that the potential risks to the environment are not justified by the 20% chance of finding enough oil to meet the needs for only 7–24 months.

3.	Improving fuel efficiency for vehicles would save more oil than might ever be obtained from ANWR.

G.	There are advantages and disadvantages to the use of conventional crude oil as an energy source.

1.	CO2 released into the atmosphere helps promote climate change through global warming.

2.	The CEO of ARCO Oil stated in 1999 that “we are embarked on the beginning of the last days of the AGE of Oil.”

H.	Oil sand or tar sand is a mixture of clay, sand, water, and organic material called bitumen – thick, sticky heavy oil with a high sulfur content.

1.	Bitumen was created by bacterial degradation and groundwater at work on oil that had escaped from its origin.

2.	The extraction and processing of this material uses a great deal of energy, so it reduces net energy yield for the oil.

3.	Northeastern Alberta, Canada, has about three-fourths of the world’s oil sand reserves, and about one-tenth is close enough to be recovered.

4.	Use of these oil sands could reduce U.S. dependence on imports from the Middle East.

5.	This extraction process has severe environmental impacts on land and produces more water pollution, air pollution, and more CO2/unit energy than conventional crude oil.

6.	Oil shale deposits may be another potential source of oil. The material in this shale is kerogen. It is estimated that there are 240 times more global supplies than for conventional oil. At present, it costs more to produce than the fuel is worth.

17-3 	Natural Gas

A.	Natural gas consists mostly of methane with small amounts of heavier hydrocarbons and a small amount of hydrogen sulfide.

1.	Conventional natural gas lies above most reservoirs of crude oil.

2.	Natural gas is sometimes burned off as an unwanted by-product of oil drilling, a waste of an energy source.

3.	Unconventional natural gas is found in other underground sources. Methane hydrate deposits are about twice as abundant as the earth’s oil, natural gas, and coal resources combined.

4.	Extraction techniques are too expensive at present, but are rapidly being developed. Methane hydrates must be kept cold, or they release methane into the atmosphere when they reach the surface.

5.	Propane and butane gases are liquefied from a natural gas field and removed as liquefied petroleum gas (LPG) that is stored in pressurized tanks.

6.	Methane is dried of water, cleaned, and pumped into pressurized pipelines for distribution.

B.	Natural gas is a versatile fuel. It can be used to run fleet vehicles and medium-sized turbines to produce electricity. They are cheaper to build, require less time to install, and easier and cheaper to maintain than coal and nuclear power plants.

C.	Russia and Iran have about 50% of the world’s reserves of conventional natural gas, and global reserves are expected to last 62–125 years.

1.	The long-term outlook for natural gas supplies is better than for conventional oil.

2.	Natural gas use should increase because it is fairly abundant and has lower pollution and CO2 rates/unit of energy compared to other fossil fuels.

D.	Natural gas was burned to provide about 53% of the heat in U.S. homes and 16% of the country’s electricity.

1.	U.S. production of natural gas is declining, and a reversal does not seem probable.

2.	Canadian imports are possible, but Canadian production is expected to peak between 2020 and 2030.

3.	Shipping of LNG is very expensive and reduces net energy yield. It is also flammable and could lead to large-scale fires at receiving terminals.

17-4 	Coal

- Coal is a solid fossil fuel formed 300–400 million years ago. It is mostly carbon with small amounts of sulfur and trace amounts of mercury. Burning coal releases SO2 and trace amounts of mercury and radioactive materials.

  - Anthracite is the most desirable type of coal because of its high heat content and low sulfur. It is less common.

  - Coal is extracted underground in dangerous circumstances (accidents and black lung disease).

  - Area strip mining is used to extract coal close to the surface. Scars from this mining are rarely restored after mining is finished.

  - Mountaintop mining has polluted 470 miles of West Virginia’s streams and displaced thousands of families.

  - Coal is usually transported by trains.

- Coal is burned to produce electricity and steel. Reserves in the U.S., Russia, and China could last hundreds to thousands of years.

  - Sixty-two percent of the world’s electricity is produced by burning coal. It is the world’s most abundant fossil fuel.

  - The U.S. has one-fourth of the world’s proven coal reserves. Russia has 16%, and China has 12%.

  - Coal reserves in the U.S. and in China should last for about 300 years at current consumption rates.

  - If coal consumption in the U.S. increases by 4% a year – as the industry projects – the reserves would last only 64 years.

  - Coal has a severe environmental impact on air, water, and land, and over one-third of the world’s annual CO2 emissions come from coal.

  - Coal emissions cause thousands of premature deaths, at least 50,000 cases of respiratory disease, and several billion dollars of property damage.

- Coal can be converted to synthetic natural gas (SNG) by coal gasification or in to liquid fuel by coal liquefaction.

  - These procedures require that 50% more coal be mined and will add 50% more CO2 emissions to the atmosphere.

  - These techniques are not possible without huge government subsidies.

  - A consortium of major oil companies is working on ways to reduce CO2 emissions during the coal gasification process; this could mean that it would be a cheaper, cleaner way to produce electricity.

17-5 	Nuclear Energy

- In a conventional nuclear reactor, isotopes of uranium and plutonium undergo controlled nuclear fission, and the resulting heat is used to produce steam that spins turbines to generate electricity.

  - A controlled chain reaction occurs when nuclei of atoms are split. The heat from the reactions is used to produce high-pressure steam that spins turbines that generate electricity.

  - Light-water reactors (LWRs) produce about 85% of the world’s nuclear-generated electricity.

  - Long, thin rods are packed with fuel pellets, and each pellet contains energy equivalent to 1 ton of coal or 4 barrels of crude oil.

  - Control rods absorb neutron-absorbing materials and move in and out of spaces between the fuel assemblies in the core. This regulates the rate of fission and amount of power the reactor produces.

  - A moderator (material that slows down neutrons) keeps the reaction going. It may be water, graphite, or deuterium.

  - A coolant, usually water, circulates through the core to remove heat to keep the components from melting and to produce steam for generating electricity.

  - A containment vessel with thick, strong walls surrounds the reactor as a safety backup. These are usually made of 4-foot reinforced concrete with a steel liner.

  - Spent rods are stored on-site in water-filled pools or dry casks with thick steel walls. These rods must be stored safely for 10,000–20,000 years until radioactivity levels are safe.

  - All the safety features make nuclear power plants very expensive to build and maintain.

- The nuclear fuel cycle includes the mining of uranium, processing it to make a satisfactory fuel, using it in the reactor, storing highly radioactive wastes safely, and dealing with the reactor after its useful life.

  - A nuclear power plant must be decommissioned after 15–60 years of operation. It contains large quantities of radioactivity that must be kept out of the environment.

  - A closed nuclear fuel cycle removes fissionable isotopes uranium-235 and plutonium-239 for reuse as nuclear fuel. This is rarely done currently because of high costs and potential use of the materials in nuclear weapons.

  - In an open nuclear fuel cycle, the isotopes are eventually buried in an underground disposal facility.  These wastes must be stored for about 240,000 years.

- Nuclear power has not lived up to its promise.

  - Nuclear power plants began being developed in the late 1950s for three reasons.

    - The Atomic Energy Commission promised utility executives that fuel would be produced at much lower costs than coal, etc.

    - The government paid about one-fourth of the cost of building the first reactors with a guarantee of no cost overruns.

    - Congress passed the Price-Anderson Act to protect the U.S. nuclear industry and utilities from significant liability in case of accidents.

  - The goals set forth in the fifties have not been met even with an investment of $2 trillion worldwide.

  - Electricity production from nuclear power plants is the slowest growing energy source.

  - The U.S. has not ordered any new reactor since 1978; all 120 plants ordered since 1973 have been cancelled.

  - Several major reasons for the failure of nuclear power to grow are multibillion-dollar cost overruns, higher operating costs, more malfunctions than expected, and poor management.

  - Public concerns about safety and stricter government regulations are two major setbacks.

  - Investors are concerned about the economic feasibility of nuclear power.

  - Vulnerability of these plants to terrorist attacks is another concern.

- The advantages of conventional nuclear fuel are: fairly low environmental impact and low risk of accident; however, costs are high, storing wastes is difficult and costly, and facilities are vulnerable to terrorist attack.

  - Increased use of nuclear fuel plants would not significantly reduce dependence on oil, since oil use is primarily for gasoline and diesel fuel for transportation.

  - The U.S. Nuclear Regulatory Commission (NRC) estimates that there is a 15–45% chance of a complete core meltdown at a U.S. reactor during the next 20 years. Also, 39 U.S. reactors have an 80% chance of failure in the containment shell from a meltdown or explosion of gases inside containment structures.

  - There is widespread distrust of the ability of the NRC to enforce nuclear safety in nuclear facilities.

- There is great concern about the vulnerability of U.S. nuclear power plants and the nuclear wastes they store to terrorist attack.

  - The 2002 study by the Nuclear Control Institute found that the plants were not designed to withstand the crash of a large jet.

  - Insufficient security at nuclear plants is another concern. The same study also found that many security guards at nuclear power plants have low morale and are overworked, underpaid, undertrained, and not equipped to repel a serious ground attack by terrorists.

- Spent fuel rods stored outside of the containment shells at nuclear plants are vulnerable to attack.

  - The storage pool generally holds 5–10 times more than the core inside a plant’s reactor.

  - If the radioactive material were exposed to air and steam, it would cause the outer cover to catch fire and burn fiercely. The fire could not be put out and would release significant amounts of radioactive materials into the atmosphere and contaminate large areas for decades.

  - Studies indicate that about 161 million people live within 75 miles of an aboveground spent-fuel site.

  - Nuclear power officials feel that plants are save from attack, while critics call for constructing much more secure structures to protect spent-fuel storage sites.

- Low-level radioactive waste must be stored safely for 100–500 years.

  - Low-level radioactive wastes give off small amounts of ionizing radiation.

  - These wastes include tools, building materials, clothing, glassware, and other materials that have been contaminated.

  - From the 1940s to 1970, most low-level radioactive waste was put in steel drums and dumped into the ocean.

  - Today in the U.S., low-level wastes are placed in drums and shipped to the two regional landfills run by federal and state governments.

  - To lower costs, nuclear industry and utility officials are lobbying Congress and the NRC to let these wastes be mixed with ordinary trash and deposited in landfills.

- Scientists cannot agree on whether there is a safe method to store high-level radioactive waste.

  - Some of the proposed methods are:

    - Bury it deep underground.

    - Shoot it into space or into the sun. This strategy has been abandoned for now.

    - Bury it under the Antarctic ice sheet or the Greenland ice cap. This strategy is prohibited by international law. There is the possibility of heat making the ice sheets unstable.

    - Dump it into descending subduction zones in the deep ocean. Wastes might be spewed out by volcanic activity. This method is also prohibited by international law.

    - Bury it in thick deposits of mud on the deep-ocean floor in areas that tests show have been geologically stable for 65 million years. Because of corrosion problems this method is also prohibited by international law.

    - Change it into harmless, or less harmful, isotopes. There is no way to do this at present.

- There is disagreement over the decision to store high-level nuclear wastes at an underground storage site in Nevada.

  - This facility is expected to cost $58 billion to build and is scheduled to open in 2010.

  - After it is filled, the facilities will be monitored for 300 years, and then sealed.

  - The Yucca Mountain site is located in an area that currently receives only 15 centimeters of rain per year.

  - There is concern that rock fractures and tiny cracks may allow water to leak into the site and corrode the containers, releasing radioactive material. This material could explode.

  - The Yucca Mountain site has a nearby active volcano, and 32 active earthquake faults running through the site.

  - In January 2002, the U.S. energy secretary stated that the site was scientifically sound despite the concerns of scientists.

  - The shipment of radioactive materials across the country makes them vulnerable to terrorist attack.

  - The U.S. National Academy of Sciences, in 2002, urged the U.S. government to slow down and rethink the nuclear waste storage process.

  - Nevada is still fighting the project in the courts.

- A nuclear power plant must be decommissioned when it reaches the end of its useful life.

  - Scientists have proposed three ways to decommission plants.

    - Dismantle the plant, and store its large volume of radioactive materials in a high-level waste storage facility.

    - Put up a physical barrier around the plant, and set up full-time security for 30–100 years before the plant is dismantled.

    - Enclose the entire plant in a tomb that must last and be monitored for several thousand years.

  - Decommissioning adds to the total costs of nuclear power as an energy option.

  - Congressional auditors reported that the owners of the nuclear power reactors are not setting aside enough money to decommission plants. Taxpayers will be saddled with the bill.

- Dirty radioactive bombs are made of explosives and mixed with radioactive material.

  - Small amounts of radioactive material may be stolen from hospitals, university laboratories, and some industries that use small amounts of radioisotopes.

  - A dirty bomb could kill 12–1,000 people and increase cancers.

  - Cleaning up such an area would cost billions of dollars.

- Nuclear power is an expensive way to produce electricity, even with government subsidies.

  - Articles in Forbes and the Economist stated that not a single nuclear power plant in the world made commercial sense.

  - Although the operating costs of many U.S. nuclear power plants have dropped in recent years, the cost of the whole nuclear fuel cycle must be considered in the true cost of nuclear power.

  - Second generation, smaller, advanced light-water reactors are supposed to have built-in passive safety features that make explosions/radioactive emissions almost impossible.

  - Nucleonics Week states that experts are unconvinced that the goals have been achieved, and it still does not eliminate the threats and hazards of long-term storage of wastes.

  - Each new plant will cost up to $2 billion.

- Nuclear breeder reactors have been very costly, with bad safety records, and the technology has essentially been abandoned.

  - Breeder nuclear fission reactors generate more nuclear fuel than they consume. This means that the world’s known uranium reserves would last at least 1,000 years.

  - The reactor uses liquid sodium coolant that ignites when exposed to air and is explosive if it contacts water.

  - The U.S. ended government-supported research for breeder technology in 1994.

  - The French built a commercial-size breeder reactor in 1986. It cost so much that it was shut down in 1998 permanently.

- Nuclear fusion has a number of advantages, but after 50 years, this technology is still at a laboratory stage.

  - This type of energy production has a number of advantages such as no emissions of conventional air pollutants or carbon dioxide. The wastes are much less radioactive so would only need to be stored for about 100 years.

  - There would be no risk of a meltdown or risk from terrorist attack.

  - Fusion power could be used to destroy toxic wastes, supply energy, and decompose water and produce hydrogen gas to run a hydrogen economy by the end of this century.

  - Building a fusion reactor would be much more expensive than the cost of a conventional reactor.

- There is disagreement over whether the U.S. should phase out nuclear power or keep this option open.

  - Some analysts feel that nuclear power should be phased out regarding all or most government subsidies and the money used to subsidize and accelerate the development of other promising energy technologies.

  - According to investors and the World Bank analysts, conventional nuclear power simply cannot compete in today’s energy market.

  - Proponents of nuclear power feel that governments should continue funding research and development.  They say that we need to keep nuclear options open if various renewable energy options fail to keep up with electricity demands and reduce CO2 emissions to acceptable levels.

# Summary

- Alternative energy sources are evaluated based on the availability of each source in the future, the source’s energy yield, cost to develop and use the source, the extent of subsidies needed, effects on national and global economic and military security, and its environmental effects.

- The advantages of oil include a supply for the next 90 years, low cost, high net energy yield, easy transportation, low land use, well-developed technology, and an efficient system of distribution.  Disadvantages include a need for a substitute discovery; low price encourages waste, air pollution, and water pollution.

- The advantages of natural gas include plentiful supplies, high net energy yield, low cost, less air pollution than oil, moderate environment impact, and easy transport. Disadvantages include the fact that it is a nonrenewable resource, release of carbon dioxide when burned, leaks, and requirement of pipelines.

- The advantages of coal include plentiful supplies, high net energy yield, low cost, well-developed technology, and air pollution can be managed with appropriate technology. Disadvantages include very high environmental impact, land disturbance, air and water pollution, threat to human health, high carbon dioxide emissions, and release of radioactive particles and mercury.

- The advantages of nuclear power include large fuel supply, low environmental impact, low carbon dioxide emissions, moderate land disruption and use, and low risk of accidents. Disadvantages include high cost, low net energy yield, high environmental impact in case of accident, catastrophic accidents, long-term storage of radioactive waste, and nuclear weapons.

## Test Items

See Instructor’s Manual, Section Two, p. 512.

More Depth: Conceptual Term Paper Topics

1.	Oil and natural gas: oil prices and economic development in developing countries; enhanced oil-recovery techniques; shale oil extraction; petrochemicals; heavy oils from Athabascan tar sands; Alaska’s Prudhoe Bay gas deposits.

2.	Coal: low-sulfur coal reserves in the United States; geographic distribution of coal-burning power plants in the United States; fluidized-bed combustion; the U.S. Synthetic Fuels Corporation.

3.	Nuclear fission: centralized energy planning in France; genetic damage to A-bomb survivors; Three Mile Island; Chernobyl; how nuclear fuel assemblies are made; radioactive tailings as a health hazard; geologic repositories for high-level radioactive wastes; commercial low-level nuclear waste dump sites; storing high-level liquid wastes; geographic distribution of nuclear power plants in the United States; keeping weapons-grade nuclear materials “out of the wrong hands”; nuclear reprocessing plants.

4.	Breeder nuclear fission and nuclear fusion.

# More Breadth: Interdisciplinary Activities and Projects

1.	Arrange a class excursion to a coal-burning power plant in your vicinity. Have a company spokesperson explain the electricity-generating process and the design and operating features of equipment and systems that control air pollution emissions and reduce thermal water pollution.

2.	Schedule a guided tour to a nuclear power plant for your class.

3.	If there is a nuclear power plant operating in your vicinity, invite a spokesperson from your local emergency disaster preparedness agency to present a guest lecture explaining the emergency evacuation plan for this facility.

Multisensory Learning: Audiovisuals

Energy Supply; 36 min.; nonrenewables and renewables; BFF.

Energy Today and Tomorrow; 1994; 23 min.; petroleum extracting distilling, wind power; FHS.

Energy: The Fuels and Man; 1978; 23 min.; what energy is; past and present uses; renewable and nonrenewable; NG.

Energy; 1994; 23 min.; sun, electricity, and hydrogen; FHS.

Fueling the Future: Running on Empty; 1988; 58 min.; traces how the automobile became the mainstay of American culture; VP.

Introduction to Petroleum in Michigan; 1987; 12 min.; geology and drilling of oil and gas formations in Michigan; BP.

New Energy Sources; 1994; 23 min.; natural gas, tides, biomass; FHS.

Oil on Ice. 2004. 90 min. DVD. GPF.

The Power Struggle; 1986; 58 min.; problems of relying on nonrenewable energy sources and strategies to create a more reliable energy future; BFF.

Toast; 12 min.; documents fossil-fuel inputs to make toast; BFF.

Nuclear Energy

A Question of Power; 1986; 58 min.; perspectives on Diablo Canyon in California and applications to the present; VP.

Bound by the Wind; 1991; 40 min.; international impact of nuclear testing on people who live downwind; VP.

Breaking the Nuclear Chain; 1984; 28 min.; tracing how uranium and plutonium become fuel rods for nuclear power plants and atomic weapons through disposal issues; GPI.

Chernobyl: Chronicle of Difficult Weeks; 1986; 54 min.; directed by Vladimir Shevchenko, two weeks to three months after the incident; VP.

Chernobyl: The Taste of Wormwood; 1987; 52 min.; on-site photography of the Chernobyl blast site with interviews; FHS.

Deafsmith: A Nuclear Folktale; 1990; 43 min.; citizens’ attempts to defend the land from nuclear pollution; VP.

Desperate Measures; 1983; 34 min.; documentary of Greenpeace’s efforts to halt nuclear waste dumping in the Atlantic Ocean; GPI.

Environmental Illness: Bad Chemistry; 60 min.; people who cannot tolerate common chemicals of modern life; FHS.

Fusion: Work in Progress; 1989; 25 min.; NG.

Nuclear Bombs in Our Future; 1994; 28 min.; the spread of nuclear weapons; political motivations of countries seeking nuclear weapons; VP.

Nuclear Dynamite. 2001. 72 min. BFF.

Nuclear Energy: The Question Before Us; 1981; 26 min.; how a nuclear reactor works; pros and cons of nuclear power; NG.

Nuclear Energy; 1994; 23 min.; a nuclear reactor, safety, radioactive wastes; FHS.

Nuclear Energy; 1998; 23 min.; a modern reactor, nuclear safety, radioactive wastes; FHS.

Nuclear Technology; 1994; 23 min.; fission and fusion, food irradiation; FHS.

Radioactive Waste Disposal: The 10,000-Year Test; 1994; 50 min.; the Nevada desert and computer simulations; FHS.

Radioactive Waste Disposal: The 10,000-Year Test; 50 min.; FHS.

The Four Corners—A National  Sacrifice Area?; 1983; 59 min.; cultural and ecological impact of energy development in the American Southwest; BFF.

The Nuclear Dream; 1991; 17 min.; examination of the failed promise of nuclear power; GPI.

The River That Harms; 1987; 45 min.; documentary of the largest radioactive waste spill in U.S. history and its impact on the Navajo Indians of New Mexico; VP.

The Transportation of Nuclear Materials; 40 min.; FHS.

Three Mile Island Revisited; 1993; 30 min.; challenges the claims of the nuclear industry and government that “no one died” from the core meltdown in 1979; evidence of cancer deaths and birth defects is presented; VP.

Tjernobyl Efteraret (Chernobyl Autumn); 1987; 29 min.; Chernobyl’s effect on Lapland; SF.

Understanding Energy.  1998.  FHS.

CD-ROM

Chernobyl: The Dead Zone. 1998. Video, medical statistics, charts, hospitals, glossary, maps, music; FHS.

Understanding Energy. 1998. Science, history, technology, and geography of energy; FHS.

See Appendix A for suppliers.

ATTITUDES/VALUES

Assessment

1.	Do you favor requiring all cars to get at least 21 kilometers per liter (50 miles per gallon) and vans and light trucks to get at least 15 kilometers per liter (35 miles per gallon) of gasoline within the next ten years?

2.	Would you favor much stricter, twice-a-year inspections of air pollution control equipment on motor vehicles and tough fines for not keeping these systems in good working order?

3.	Would you favor a $2 tax on a gallon of gasoline and heating oil to help reduce wasteful consumption, extend oil supplies, reduce air pollution, delay projected global warming, and stimulate improvements in energy efficiency and the use of less harmful energy sources?

4.	Would you vote for anyone proposing a program to add a $2 per gallon tax on gasoline and heating oil, assuming all other factors are the same?

5.	Would you support laws requiring that all new homes and buildings meet high energy-efficiency standards for insulation, air infiltration, and heating and cooling systems?

6.	Would you favor such a law for existing homes and buildings?

More Depth: Discussion and Term Paper Topics

1.	Do you feel that the application process for nuclear power plants should be streamlined to limit citizen input?

2.	Do you think emphasizing nuclear power and oil as the primary U.S. energy sources shows adequate concern for future generations?

3.	Whistle-blowers. The Karen Silkwood story.

(Also, see text, Critical Thinking, p. 378 and Critical Thinking and the Environment.)

PARTICIPATION

Lifestyle and Campus Community

See Green Lives/Green Campuses, Chapter 10: Generating Heat; Chapter 11: Generating Electricity; Chapter 12: Energy: A Generation on the Go.

More Depth: Action-oriented Term Paper Topics

1.	National. The Gold Rush; the Mining Law of 1872; The Atomic Energy Commission (1946–1975); the Nuclear Regulatory Commission; the Energy Research and Development Administration (ERDA); the Nuclear Safety Analysis Center; the Institute of Nuclear Power Operations; the Price-Anderson Act.

2.	Global. The relationship between the United States and countries with large supplies of critical and strategic minerals; the relationship between developing countries and developed countries regarding mineral supplies; manganese nodules on the ocean floor.

SKILLS

Environmental Problem-solving Skills: Projects

1.	As a class, conduct a school and community survey to assess people’s beliefs about and attitudes toward America’s reliance on foreign sources for various strategic materials.

2.	As a class exercise, assign appropriate roles and research assignments to students (or teams of students) and stage mock negotiations among several less-developed nations seeking to establish a cartel for controlling the supply and price of the vitally important industrial commodity X. Assume that the cartel is successful and stage additional mock negotiations between the cartel and more-developed nations whose economies are suffering grievously for want of affordable supplies of commodity X.

3.	As a class exercise, (a) obtain cost estimates for the construction of a single large-scale synfuels plant and (b) calculate how many soft path energy facilities or systems of various types (such as solar water heaters, biogas digesters, and photovoltaic devices) could be installed with an equivalent amount of money.

4.	As a class exercise, make a list of the geopolitical responsibilities and costs incurred by the United States in association with maintaining an uninterrupted supply of affordably priced oil from foreign sources.

5.	A new power plant must be built in your community, but it remains to be decided whether it will be a fossil-fuel or nuclear plant. As a class exercise, set up a mock public hearing to present the arguments for both sides. Make specific role assignments so that prepared statements will accurately reflect varying points of view such as those of contractors, environmentalists, project engineers, state energy officials, and concerned citizens.

Laboratory Skills

Laboratory Manual for Miller’s Living in the Environment and Environmental Science. Lab 19: Energy Conservation.

Computer Skills

ENERPLAN

-Performs basic energy analysis for a nation, province, or community.

-Mr. Nicky Beredjick, Director, National Resources and Energy Division, Department of Technical Cooperation for Development, United Nations, New York, NY 10017

Estimating Fossil Fuel Resources (EFFR)

-Simulates the global exploitation of oil resources and evaluates alternative resource-estimation techniques.

-Prof. John D. Sterman, Sloan School of Management, Massachusetts Institute of Technology, 50 Memorial Dr., Cambridge, MA 02139

Also, see Introduction to the Internet.